BEP Template

BIM Execution plan

Pilot Project TBC SP Energy Networks

Note:

Several sections of this document will be updated following the BIM kick-off meeting, creation of the 3D information models or are awaiting document references. In each of these cases, the content has been highlighted in orange.

Contents

1	INTRODUCTION	3

1.1	Purpose of document	3

1.2	Amendment of documents	3

2	PROJECT INFORMATION	4

2.1	General	4

2.2	Programme and phase information	4

3	APPLICABLE STANDARDS	1

4	PROJECT FUNCTIONS	1

4.1	Project Team BIM organisation	1

4.2	Delivery Team composition	2

4.3	Information Management function	2

4.4	Construction Delivery Team Capability & Capacity Summary	7

4.5	Training Requirements	8

5	DETAILED (BIM) RESPONSIBILITY MATRIX	9

5.1	Detailed (BIM) Responsibility Matrix	9

5.2	MIDP	9

6	INFORMATION DELIVERY STRATEGY	10

6.1	Approach to meeting the appointing party’s exchange information requirements	10

7	INFORMATION STANDARD	17

7.1	Exchange of information between task teams	17

7.2	Delivery of information to the Appointing Party	19

7.3	Identification (ID) conventions	23

7.4	Classification of information	26

7.5	Risk Register	27

7.6	Task information Delivery Plan	27

8	INFORMATION PRODUCTION METHODS AND PROCEDURES	28

8.1	Common Data Environment	28

8.2	Collaboration process	29

8.3	Generation, review, approval and authorisation of information	32

8.4	Security and distribution of information	33

8.5	Legacy information	36

9	FEDERATION STRATEGY	37

9.1	Information breakdown strategy	37

9.2	Federation process	37

10	SOFTWARE, HARDWARE AND IT INFRASTRUCTURE	38

10.1	Software	38

10.2	Hardware and IT Infrastructure	38

Appendix A – Abbreviations and glossary of terms	39

1.	Abbreviations	39

2.	Glossary of terms	40

Appendix B - Reference summary	42

Appendix C - Detailed (BIM) Responsibility Matrix	45

Appendix D – Master Information Delivery Table	46

Appendix E – Asset Information Requirements	47

Appendix F – IFC Export Settings	48

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

| Revision | Status | Originator | Approver | Date |
| --- | --- | --- | --- | --- |
| P01 | S2 | Niall Murphy |  | 30/04/2021 |
| © Turner & Townsend Consulting Limited. This document is expressly provided to and solely for the use of SP Energy Networks on the Pilot Project TBC and takes into account their particular instructions and requirements. It must not be made available or copied or otherwise quoted or referred to in whole or in part in any way, including orally, to any other party without our express written permission and we accept no liability of whatsoever nature for any use by any other party. |  |  |  |  |

# INTRODUCTION

## Purpose of document

This document explains how the information management aspects of Pilot Project TBC will be carried out by the delivery team. In doing so this document will respond to the Appointing Party’s EIR (doc reference) and confirm compliance as required with the ISO 19650 series and remaining PAS 1192 series specifications. All members of the Delivery Team are required to adopt the agreed standards, methods and procedures contained throughout this document.

Prior to the commencement of the project’s Construction Stage, the Construction Lead (Appointed Party) shall develop this BIM Execution Plan to generate the Construction Delivery Team BIM Execution Plan and maintain this document for the remaining project duration. The Construction Lead (Appointed Party) shall incorporate existing processes identified within the design phase BEP into their own construction phase BEP to mitigate the risk of information gaps arising.

## Amendment of documents

Both the EIR and the BEP may be subject to amendments and further developments during the execution of the project.

Once the EIR is issued as part of the ITT packs, any amendments to the information requirements defined in the EIR will be captured and agreed within the Construction Delivery Team BIM Execution Plan. The BEP shall be considered a live document, capturing continuous improvements throughout the construction phase of the project.

# PROJECT INFORMATION

## General

| Appointing Party | SP Energy network (SPEN) |
| --- | --- |
| Appointing Party Main contact | TBC |
| Project name | Pilot Project TBC |
| Short project description | TBC |
| Project address | TBC |
| Correspondence address | TBC |
| Plan of works | TBC |

Table 1. General Project Information

## Programme and phase information

As per the Exchange Information Requirements (EIR) this BIM Execution plan expands upon the phase specific programme information. Details of this are as follows;

Organisations, individuals involved, and their assigned functions are outlined in section 4

Date of completion of all project stages shall be in accordance with the project programme (doc reference), which is maintained separately from this document

Dates of project milestones including Appointing Parties’ information exchanges are outlined relative to project stages within section 7.2.1

# APPLICABLE STANDARDS

As communicated by the EIR, SP Energy Networks requires Project Stakeholders to adopt the following references.

|  |  | Application |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference M: Mandatory R: Recommended | Collaboration | Project stages | Information container naming | Object authoring | Drawing | Classification | Level of information need | Common Data Environment | Asset Information | Appointments | Facilities Management | Health & Safety | Costing | Security | Survey |  |
| Core Standards | BS EN ISO 19650-1:2018 | M |  |  |  |  |  | M | M |  | M | M |  |  |  |  |  |
|  | BS EN ISO 19650-2:2018 | M | M | M |  |  |  | M | M |  | M |  |  |  |  |  |  |
|  | BS EN ISO 19650-3:2020 |  |  |  |  |  |  | M | M | M | M | M |  |  |  |  |  |
|  | BS 1192-4:2014 | M |  |  |  |  | M | M |  | M |  | M |  |  |  |  |  |
|  | BS EN ISO 19650-5:2020 | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M |  |
|  | PAS 1192-6:2015 |  |  |  |  |  |  |  |  | M |  | M | M |  |  |  |  |
| Enablers | BS 8541-1:2012 |  |  |  | M | M |  |  |  |  |  |  |  |  |  |  |  |
|  | BS 8541-2:2011 |  |  |  | R | R |  |  |  |  |  |  |  |  |  |  |  |
|  | BS 8541-3:2012 |  |  |  | R | R |  |  |  |  |  |  |  |  |  |  |  |
|  | BS 8541-4:2012 |  |  |  | R | R |  |  |  |  |  |  |  |  |  |  |  |
|  | BS 8541-5:2015 |  |  |  | R | R |  |  |  |  |  |  |  |  |  |  |  |
|  | BS 8541-6:2015 |  |  |  | R | R |  |  |  |  |  |  |  |  |  |  |  |
|  | NBS Definition Library |  |  |  | M | M |  | M |  |  |  |  |  |  |  |  |  |
|  | Uniclass 2015 |  |  |  | M |  | M |  |  |  |  |  |  |  |  |  |  |
|  | BS 5964-3:1996 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | M |  |
| Project Protocols | Information Management Protocol | M |  | M |  | M |  |  | M |  |  |  |  |  |  |  |  |
|  | BIM Execution Plan (BEP) | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M |  |
|  | Exchange Information Requirements (EIR) | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M |  |

Table 2: Applicable standards

Further explanation on the references is included in Appendix B.

# PROJECT FUNCTIONS

## Project Team BIM organisation

Project Team

Appointing Party

Appointing Party

Information Managers

Delivery Team

SP Energy Networks

Appointing Party

TBC

Lead Appointed Party Information Manager

TBC

Lead Appointed Party CDE Information Manager

Design Delivery Team

Designer B

Appointed Party

Designer C

Appointed Party

Designer D

Appointed Party

Designer E

Appointed Party

Designer n

Appointed Party

Designer A

Appointed Party

TBC

Design Lead (Appointed Party)

Construction Delivery Team

Sub-contractor B Appointed Party

Sub-contractor C

Appointed Party

Sub-contractor D

Appointed Party

Sub-contractor E

Appointed Party

Sub-contractor n

Appointed Party

Sub-contractor A

Appointed Party

TBC

Construction Lead (Appointed Party)

Figure 01 below outlines the project team BIM organisation for Pilot Project TBC for the overall phases of the project. Figure 01 will be updated as parties join the project team. It is important to note this diagram does not act to identify contractual relationships.

Figure 01. Project Team BIM organisation

## Delivery Team composition

The table below outlines the constituent individuals of each Task Team.

| Individual | Email | Telephone |
| --- | --- | --- |
| Task Team A (name) |  |  |
| TBC | TBC | TBC |
| TBC | TBC | TBC |
| Task Team B (name) |  |  |
| TBC | TBC | TBC |
| TBC | TBC | TBC |
| … |  |  |

## Information Management function

The table below outlines project information management functions and responsibilities on Pilot Project TBC. To provide further clarity and avoid scope gaps a supporting RACI table is included in Appendix F of the EIR.

The functions and responsibilities outlined in Table 3Error! Reference source not found. below should not be confused with job titles or other professional designations within the project team. Additionally, the responsibilities outlined below are to be considered ‘key’ responsibilities associated with the project function as outlined in the BS EN ISO 19650 series. Any further responsibilities documented within the main body of this document are also to be fulfilled.

| Function | Individual / Party fulfilling function | Responsibilities |
| --- | --- | --- |
| Appointing Party |  |  |
| Appointing Party | SP Energy Networks | Approval of the appointment of Project Stakeholders Approving changes to the master programme, budget cost, design, and information management implementation Provide a Common Data Environment Agreeing a policy for the project by final approval of the BEP Appointment of a third-party or assignment of internal resource to undertake the Appointing Party Information Management function. |
| Appointing Party Information Manager | SP Energy Networks BIM Lead | Establish the Project Information Requirements Establish the information delivery milestones Establish the project's information standard Establish the project's information production methods and procedures Creation of the EIR Review and approve the BEP Review and approve the Master Information Delivery Plan Establish a programme for the review of the Delivery Team’s risk register Review and comment on the capability of the (prospective) Lead Appointed Party to deliver against the requirement set-out herein Facilitate and chair the information management workgroup and strategy meetings Report progress, drivers and highlight risks to the information management process to SP Energy Networks and project stakeholders Monitor compliance and report against information supplied by the Delivery Team against the requirements set-out herein Capture lessons learnt in collaboration. |
| Appointing Party CDE Information Manager | TBC | Facilitate the management of the federated model and the production of project outputs Establish a CDE for the management and exchange of information models in consultation with the Appointing Party Information Manager Manage the operation, standards, and culture of the CDE Implement, configure, and maintain the CDE to align with the information management principles defined in section 7 of the EIR Author and maintain the information management protocol relating to the exchange of all project information Provide training and support to all relevant project stakeholders for use and understanding of CDE functions Verify, implement and maintain the processes and procedures for validating the Appointing Party exchange information requirements between the project team Assess and report to the Appointing Party and Appointing Party Information Manager at defined intervals the compliance of the Delivery Team’s information deliverables shared via the CDE Assess and report to the Appointing Party and Appointing Party Information Manager that the Delivery Team are utilising appropriate software and have the appropriate level of skill With Task Team Managers, establish and implement the information container structure and maintenance standards for the Project and Asset Information Model within the BEP. |
| Built Asset Security Manager | TBC | Provide a holistic view of the security issues and threats to be addressed Offer guidance and direction on the handling of risks Take ownership, manage, and assist in the development of the built asset security strategy (BASS) Be accountable for security decisions that are taken Take ownership, manage, and assist in the development of the built asset security management plan (BASMP) Take ownership, manage, and assist in the development of security breach/incident management plan (SB/IMP) Take ownership, manage, and assist in the development of the built asset security information requirements (BASIR) Assist in the development and reviewing of any tendering and project planning documentation Be responsible for promoting a security-minded culture Brief advisors, specialists and supply chain on relevant aspects of the BASS, BASMP and BASIR Advise on the need for, and undertake, the review and auditing of documentation, policies, processes and procedures relating to the security of the built asset Where appropriate and necessary, seek appropriate professional security advice to provide additional guidance throughout the lifecycle of the project and/or asset. |
| Appointed Parties |  |  |
| Design Lead (Appointed Party) | TBC | Nominate an individual to undertake the Lead Appointed Party Information Management function. Establish and maintain the BEP Establish and maintain the Design Delivery Team’s risk register associated with the timely delivery of information Compile the Master Information Delivery Plan (MIDP) through collation of the Task Information Delivery Plans generated by each Task Team Test the project’s information production methods and procedures including the CDE Collate the Appointed Parties individual detailed responsibility matrices into a master detailed BIM responsibility matrix Manage progression of the Design Delivery Teams information model against the requirements of the EIR Review and authorise each task team's information model at key milestones as defined in section 7.2 of the EIR Demonstrate the compliance of the Design Delivery Team information models for use by the Construction Delivery Team in accordance with section 7.2 of the EIR Capture lessons learnt in collaboration with the Appointing Party Facilitate and chair information model coordination meetings. |
| Construction Lead (Appointed Party) | TBC | Nominate an individual to undertake the Lead Appointed Party information management function Establish and maintain the BEP Establish the Construction Delivery Team’s capability and capacity Establish the Construction Delivery Team’s proposed mobilisation plan Establish and maintain the Construction Delivery Team’s risk register associated with the timely delivery of information Compile the Master Information Delivery Plan (MIDP) through collation of the Task Information Delivery Plans generated by each Task Team Test the project’s information production methods and procedures including the CDE Collate the Appointed Parties individual detailed responsibility matrices into a master detailed BIM responsibility matrix Manage progression of the Construction Delivery Team’s information model against the requirements of the EIR Review and authorise each Task Team's information model at key milestones in accordance with the requirements outlined in section 7.2 of the EIR Capture lessons learnt in collaboration with the Appointing Party Final handover of all project information in the agreed formats to the Appointing Party. |
| Appointed Parties | TBC | Contribute to the development of the BEP Contribute to the production of the MIDP by submitting TIDPs Contribute to the production of the risk register around the timely delivery of information Contribute to the development of the Delivery Team’s detailed BIM responsibility matrix Collaboratively generate information in compliance with the project information standards and information production methods and procedures Validate your own information containers in accordance with the requirements outlined in section 7.2 of the EIR prior to release to the Lead Appointed Party for review and authorisation. |
| Principal Designer | TBC | As defined in The Construction (Design and Management) Regulations 2015. |
| Principal Contractor | TBC | As defined in The Construction (Design and Management) Regulations 2015. |
| Task Teams Any multi-disciplinary team or team within an organisation assembled to carry out a task on the project. |  |  |
| Task Team Manager | Individuals fulfilling this function to be confirmed by each Appointed Party | Individual responsible for a specific task team The production of design outputs related to a discipline-specific package based or time-based task Issuing their team’s approved information within the CDE. |
| Interface Manager | Individuals fulfilling this function to be confirmed by each Appointed Party | Individual responsible for managing spatial co-ordination on behalf of a specific task team Managing spatial coordination on behalf of a task team Proposing resolutions to coordination clashes. |
| Task Information Manager | Individuals fulfilling this function to be confirmed by each Appointed Party | Individual within a task team responsible for production of information for a given task in compliance with defined standards and methods Directing the production of task information in compliance with agreed standards and methods defined in the BEP to enable reliable information exchange through the CDE Confirming to the Task Team Manager that information is suitable for issue within a CDE. |
| Information Originator | Individuals fulfilling this function to be confirmed by each Appointed Party | Developing constituent parts of the information model in connection with specific tasks The production of project outputs for the maintenance and receipt of information into the information model Ownership of the authored model. |

Table 3. Project Functions

## Construction Delivery Team Capability & Capacity Summary

The Appointed Party BIM assessment forms (Appendix J of the EIR) must be completed by all Appointed Parties within the Construction Delivery Team. A summary of each discipline’s capability and capacity is outlined in below. Full assessment documents are available from the Lead Appointed Party.

| Organisation | Discipline | Overall Assessment Rating | Identified Support Requirement | Mitigation Strategy |
| --- | --- | --- | --- | --- |
| TBC | TBC | TBC | TBC | TBC |

Table 4: Construction Delivery Team capability and capacity

## Training Requirements

| Project Module Name | Module Delivered to | Date Delivered |
| --- | --- | --- |
| Delivering key parts of the BIM Execution Plan | Design Team, Appointing Party | <Enter Date> |
| Clash detection process and reporting | Design Team, Appointing Party | <Enter Date> |
| Model viewer & navigation | Design Team, Appointing Party | <Enter Date> |
| CDE fundamentals | Design Team, Appointing Party | <Enter Date> |

Table 5: Training requirements

# DETAILED (BIM) RESPONSIBILITY MATRIX

## Detailed (BIM) Responsibility Matrix

For the agreed Detailed (BIM) Responsibility matrix refer to the following document.

XXX-TTD-XX-XX-SP-K-000004

Before the commencement of the project’s Build stage the Construction Lead (Appointed Party) shall further develop the matrix to reflect the delegated responsibilities of the Construction Delivery Team. To facilitate this, each Task Team within the Construction Delivery Team will provide to the Construction Lead (Appointed Party) a BIM Responsibility Matrix sufficient to address the scope of their appointment.

The Task Team Manager from each Task Team shall be responsible for providing this document and for this purpose shall use the template contained within Appendix E of the EIR. In carrying out this responsibility Task Team Managers shall comply with requirements of section 7.2 of the EIR.

## MIDP

For the Master Information Delivery Plan refer to the following document.

XXX-TTD-XX-XX-SP-K-000005

This document shall remain live throughout the project. The Lead Appointed Party is responsible for updating the MIDP as required, including on occasions when a new task team joins the Delivery Team. To facilitate this, each Task Team shall maintain their own TIDP and notify the Lead Appointed Party as the nature of the information they plan to delivery changes.

# INFORMATION DELIVERY STRATEGY

## Approach to meeting the appointing party’s exchange information requirements

### Level of information need

As per the EIR level of information need must be strictly in accordance with the ISO 19650 series and will adopt the NBS Definition Library as a benchmark for compliance. Additionally, information provided at information milestones aligned to key decision points, shall be sufficient to satisfy the Project Information Requirements (PIR).

Level of Information Need shall be understood as comprising the geometric and non-geometric aspects defined in Figure 02 below. Additionally, Level of Information Need shall be sufficient to satisfy the Project Information Requirements set out in Appendix I of the EIR. Responsibility for geometric and non-geometric information is defined by the Detailed (BIM) Responsibility Matrix as detailed in section 5.1.

Figure 02. Constituent aspects of Level of Information Need

Responsibility for geometric and non-geometric information is defined by the Detailed (BIM) Responsibility Matrix as detailed in section 5.1. Each Task Team will populate the required information whilst complying with the following;

Attribute names for asset information shall match exactly the parameter names within Appendix H of the EIR

Attribute names for Uniclass 2015 classification shall match exactly the parameter names defined in section 7.5

Attribute names for performance data shall match exactly the parameter names defined in section 6.1.3.1

Attribute names for carbon tracking shall match exactly the parameter names defined in section 6.1.3.4

Attribute names shall be used only once per information container, avoiding parameters on the same name

Where authoring applications allow for grouping of attributes, ‘Data (Type)’ shall be used for type information and ‘Data’ for instance information.

Level of Information Need shall not exceed that required

#### As built 3D information model

The Construction Lead (Appointed Party) shall update this section prior to the project’s Construction Stage to incorporate methods and responsibilities for the creation of an as built 3D information model.

### Structured asset information

Structured asset information is to be delivered in accordance with BS 1192-4:2014 (COBie) and contained within a single Information Container when issued at information exchange milestones.

The requirements for asset information are as detailed in Appendix H, section 2 of the EIR. Task Teams will populate non-geometric information for components within their design/construction scope or as otherwise defined by the Detailed (BIM) Responsibility Matrix, Appendix C where they appear as a maintainable asset within Appendix H, section 2 of the EIR.

Where a Task Team are using Autodesk Revit as their primary BIM authoring software, they will use the Autodesk BIM interoperability tool add-on to general COBie parameters for population. This add-on is available to download from the following location.

| Autodesk BIM Interoperability Tool | https://interoperability.autodesk.com/ |
| --- | --- |

It is imperative that this tool is run at the beginning of the project. To ensure information consistency The Design Lead Appointed Party shall issue a standardised setup file, made available at the following location.

| Container ID | <Enter information container identity> |
| --- | --- |
| CDE location | <Enter CDE location> |

Where a Task Team are not using Autodesk Revit, they shall manually create and populate parameters within their 3D information model(s) that match exactly the attribute names contained within Appendix H, section 2 of the EIR.

#### Transfer of design/construction responsibility between Task Teams

Where responsibility for components transfers between task teams, the following processes shall be applied as relevant.

Recipient task team makes Geometric changes: Component ID shall be updated to reflect recipient task team.

Recipient task team makes Non-geometric changes: COBie.CreatedBy attribute shall be updated to recipient task team. Component ID shall not change.

Recipient task team makes Geometric and non-geometric changes: COBie.CreatedBy attribute shall be updated to recipient task team. Component ID shall be updated to reflect recipient task team.

### Additional non-geometric information

#### Performance information

The requirements for performance information are as detailed in Appendix H, section 3 of the EIR. Task Teams will populate non-geometric information for components within their design/construction scope or as otherwise defined by the Detailed (BIM) Responsibility Matrix, Appendix C where they appear as a maintainable asset within Appendix H, section 3 of the EIR.

Task Team shall manually create and populate parameters within their 3D information model(s) that match exactly the attribute names contained within Appendix H, section 3 of the EIR.

#### Cost management information

Area and quantity calculation

| Area measurement standard | TBC |
| --- | --- |

Task teams will derive information from the 3D information models directly with no editing of naming and values in other software. Area scheduling must adhere to the following terms:

GIA – Gross Internal Area

GEA – Gross External Area.

Area information reported from the information models must be current, consistent with the design intent and in alignment with the terms above using definitions from Uniclass 2015 classifications.

#### Model authoring for use with CM software

To support costing using the 3D information model, Information Originators must adhere to the following:

All building elements must be modelled as an instance of a 3D BIM object unless specifically agreed by the Cost Manager, Appointing Party Information Manager and Lead Appointed Party

BIM objects must be authored using the correct building category, or IFC mapping to allow accurate IFC type mapping during export processes

BIM objects must be modelled in accurate locations, with accurate dimensions in alignment with the design intent

All duplicate BIM objects must be removed when shared on the CDE

Layered or composite BIM objects must contain material information (defining the materials of the BIM objects) consistent with the design intent, or must be indicated as “concept”.

Any additional non-geometrical cost management information to be associated as additional parameters to BIM objects must be agreed and defined in the BEP

Spaces must contain accurate information regarding finishes, room function, name and intended occupancy where known.

#### Carbon tracking information

The 3D information models shall assist in the tracking of embodied carbon. To support this, each Task Team is required to link their component to a numeric parameter symbolising the component’s embodied carbon. The numeric parameter shall be produced by the embodied carbon specialist.

| Carbon tracking parameter name | CarbonTrackingMetric |
| --- | --- |

### Approach to facilitating the Appointing Party’s strategic BIM priorities

To maximise the benefits of BIM, the Delivery Team will work openly to achieve the Appointing Party’s strategic BIM priorities as outlined in EIR, section 3.1.

| Appointed Party priority | Delivery team approach |
| --- | --- |
| Project delivery of the highest quality | Task Teams will meet the requirements of the EIR. This BIM Execution Plan provides a response to these requirements that is aligned with industry best practice |
| Better informed Appointing Party decision making, earlier and more efficient reporting of developing design information allowing key changes to the design to be made earlier, at less cost | Appointing Party’s key decision points have been identified and aligned against the Delivery Team’s information delivery milestones (section 7.2.1). Information that Task Teams are required to provide at each milestone has been defined Information delivery will be aligned to the Appointed Party’s stated Level of Information Need (section 6.1.1) |
| Improved multidisciplinary design coordination and reduced variation costs during construction | Collaboration process has been outlined in section 8.2. This includes provision for coordination and clash detection (section 8.2.2) to occur at regular periods through design and construction |
| - Visual communication and stakeholder engagement | 3D information models and 2D drawings will be provided to the Appointing Party at information delivery milestones outlined in section 7.2.1. 3D information models will form the basis for design team meetings |
| Improved accuracy and consistency of design information | Agreed model tolerances outlined in section 8.3.3. Lead Appointed Party’s process for information review and authorisation outlined in section 7.2.2 |
| Improved health and safety on site and during operation | Use of 3D information models to support health & safety has been outlined in section 8.4.2 |
| Asset information delivery of the highest quality | Delivery Team’s approach to delivery of non-geometric information has been outlined in sections 6.1.2 and 6.1.3.and aligns with industry best practice |
| More efficient visual communication of the design intent as this develops | 3D information models will form the basis for design team meetings |
| Improving communication and encouraging collaboration between project team members | Details of collaborative BIM meetings are provided in section 8.2.1. Process for sharing information between Task Teams is provide in 7.1, and with the Appointing Party in section 7.2 |
| Improving efficiency and reducing waste in the whole lifecycle including design, construction, operation, maintenance, decommissioning and handover of the Works | The Delivery Team will adopt a coordination and clash detection approach (section 8.2.2) to minimise the need for rework. Performance and carbon tracking information will be hosted within the 3D information models to allow the Appointed Party to monitor these attributes. The Delivery Team will provide structured asset information (section 6.1.2) to support operational and maintenance activities will |
| Improving the planning and coordination of installation and commissioning works by the use of 4D (Construction Lead (Appointed Party)) models for developing the phasing of the Works | Task Teams will model in a manner replicating the method of construction. Alternatively, where a Task Team has no construction responsibility models must be produced in such a manner as to prove buildability |
| Having a clear plan for delivering structured and quality controlled information | The delivery approach for information is outlined in sections 7.1 and 7.2 and comply with the EIR requirements |
| Having a clear view of how project software platforms interact | The software chosen by the Task Teams (section 10.1) is interoperable and supports the collaborative workflows of the Project Team. Federated models will be shared in .ifc and .nwc format. Viewers for this file format are readily available for free. |
| Developing the knowledge and skills of the project team in relation to BIM, supported with appropriate training where necessary | Appropriate training has been identified for project stakeholder and documented in section 4.5 |
| Having a long term, high quality information base which is properly maintained up to and including decommissioning and/or handover at the end of the appointment term | All stakeholders are to comply with the Project Information Protocol. Information to be provided to the Appointed Party has been defined in section 6.1 |
| Having a single source of truth for monitoring performance against agreed KPIs and providing evidence for payment. | Performance monitoring against KPIs will be checked by the Appointing Party Information Manager who will report the outcome to the Appointing Party. KPIs as appear in the EIR have been included in section 7.2.4, to enable Task Teams to carry out their own checks. |

Table 6: Approach to facilitating the Appointing Party BIM priorities

### Approach to completing the Appointing Party’s BIM uses

The Delivery Team approach to completing the Appointing Party’s BIM uses as outlined in EIR, section 3.2 is outlined in Table 7 below.

| Required BIM Use | Delivery team approach | Responsibility |
| --- | --- | --- |
| 2D drawing generation | Task Teams will generate 2D graphical information directly from the 3D information models | Information Originators |
| 3D design authoring & coordination | As per this BIM Execution Plan, Task Teams will author 3D information models. One use of these models will be to carry out individual and federated coordination checks as per section 8.2.2 | Information Originators, Task Interface Managers, Lead Appointed Party |
| 3D design reviews | The 3D information models will for the basis for design team meetings. Additionally any coordination issues arising will be discussed during BIM coordination meetings | Task Teams Managers, Task Teams |
| Buildability analysis | TBC | TBC |
| Cost estimation & management | As per section 6.1.3.3 Task Teams will facilitate the Cost Managers use of the 3D information models by following best modelling practices. As per section 6.1.3.2 Task Teams will derive any area schedules directly from the 3D information model and assigned Uniclass classification to spaces As per section 7.2.1.1 Task teams will provide information at each milestone inclusive of a .dwfx model for use by the Cost Manager. | Information Originators, Cost Manager |
| Existing condition modelling | As per sections 8.5 Task Teams will make use of existing legacy information to support project development | Task Team Manager |
| Health and safety & risk management | As per section 8.4.2 Task Teams will include ‘risk cubes’ within their models to identify and track any health & safety risks. These risks will be associated to the project’s health and safety register. | Principal Designer/Contractor, Information Originators |
| Information classification | As per section 7.4 Task Teams will apply appropriate Uniclass2015 classification to the facility, project roles, spaces and components | Information Originators |
| Model assurance & information validation | As per sections 7.1.2, 7.2.2 and 8.2.2 geometric, non-geometric and compliance checks will be undertaken by the Task Teams prior to sharing information. As per 8.2.1 BIM coordination meetings will occur regularly to address any issues that are present in the 3D information models | Task interface Managers, Lead Appointed Party |
| Embodied carbon tracking | As per section 6.1.3.4 Task Teams will populate a Carbon Tracking parameter for each component within their model | Information Originators Carbon Tracking Team |
| Planning, sequencing, & simulation | Task Teams will model in a manner replicating the method of construction or in such a manner as to prove buildability. The Construction Lead (Appointed Party) may use this information to carry out buildability checks | Information Originators, Task Team Managers, Construction Lead (Appointed Party) |
| Asset management & planned maintenance | As per sections 6.1.2 and 6.1.3 Task Teams will populate within their model(s) parameters consistent with Appendix H of the EIR | Information Originators |

Table 7: Approach to completing the Appointing Party's BIM Uses

# INFORMATION STANDARD

## Exchange of information between task teams

### Frequency of information exchange

To ensure compliance with the EIR and facilitate collaborative working processes each Task Team shall upload the native format and exchange information formats of their 3D information model on a continual fortnightly cycle, following the shared CDE workflow.

Refer to section 7.2.1.1 for details of the native format and exchange information format.

|  | Week 1 |  |  |  |  | Week 2 |  |  |  |  | Week 3… |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F |
| Development of 3D information models | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Internal QA/QC check |  |  |  | ✓ |  |  |  |  |  |  |  |  |  | ✓ |  |
| Issue information exchange to CDE |  |  |  |  | ✓ |  |  |  |  |  |  |  |  |  | ✓ |

Table 8: Schedule for information exchange between task teams

### Internal QA/QC

As required by the EIR, each Task Team shall undertake a quality assurance check of each of their information models, in accordance with the project’s information production methods and procedures, prior to being shared to the CDE. Responsibility for quality assurance checking lays with the Task Information Manager, however as appropriate the aspects relating to preparation of the model for sharing may be delegated to an Information Originator within the Task Team.

In carrying out the quality assurance, the Task Team shall check the Information Model in accordance with the project’s information standard, and must check that:

All linked information models and links to centralised information sources or databases have been removed

All extraneous drawing sheets (i.e. those deemed to not be a deliverable) have been removed from the Information Model

All linked information models contained within the Information Model but not present in the design have been removed

Information Model format and Information Container identification (ID) conform to this document and the BEP and remain constant for the life span of the project

Federation strategy conforms to project requirements

The Information Model and 2D drawings are up to date and the 2D information has been derived from the Information Model with no modification in 2D modelling software

All BIM objects in the Information Model present in the design have been made visible

All ownership of any shared worksets has been relinquished

All information models are using the shared common coordinate system defined at the outset of the project

All construction items to be represented in 3D as defined in the level of information need requirements of the Detailed BIM Responsibility Matrix

There must be no space gaps.  Bounding boxes used to represent spaces and zones must match with architectural requirements and values and must not conflict

Spatial information must be generated and associated with bounding elements (walls, doors, windows, floors, columns, ceilings)

Modelling must follow the method of construction. Alternatively, where a Task Team has no construction responsibility models must be produced in such a manner as to prove buildability.

Figure 03. Internal QA/QC process

Once the quality assurance check is complete the Task Information Manager shall notify the Task Team Manager of the outcome. If the check is passed the Task Team Manager shall assign the correct status code to the Information container and upload to the CDE using the shared CDE workflow.

If the internal assurance check is not passed the Task Team shall rectify the identified issues and repeat the check. The Task Team Manager shall notify the Lead Appointed Party if the issues can not be resolved before the schedule date of information exchange.

### Referencing of 3D information models

Where a Task Team references 3D information model(s) produced by another Task Team they shall immediately update their reference upon access to a more recent version of the 3D information model.

## Delivery of information to the Appointing Party

### Information Delivery milestones

As per the EIR, timing regarding information exchange requirements shall be referred to as information delivery milestones. Information exchanges must be supplied to the Appointing Party at each information delivery milestone.

Table 9 below outlines the agreed information delivery milestones aligned to project work stages or asset management requirements.

| Type | Information Delivery Milestone | Information Delivery Date | Key Decision Point? | CDE Workflow | File formats as per section… |
| --- | --- | --- | --- | --- | --- |
| Continual | Fortnightly all stages | TBC | No | Shared |  |
| Singular | Midpoint of Tender Design Stage | TBC | No | Shared |  |
| Singular | End of Tender Design Stage | TBC | Yes | Published |  |
| Singular | Midpoint of VFC Design Stage | TBC | No | Shared |  |
| Singular | End of VFC Design Stage | TBC | Yes | Published |  |
| Continual | Monthly throughout Construction stage | TBC | No | Published |  |
| Singular | End of Construction stage | TBC | Yes | Published |  |

Table 9: Information delivery milestones

#### Information to be provided at each information milestone

The following information is required for each information delivery milestone:

| Information | Format (all listed are required) |
| --- | --- |
| All milestones |  |
| 3D information model | native format, e.g .rvt .nwc |
| Key decision points |  |
| 3D information model (additionally) | .ifc 2x3 (export settings defined in Appendix F) .dwfx (subject to change as cost management software is confirmed) |
| Response to Project Information Requirements | As appropriate to address PIR |
| Non-geometric information Structured asset information Performance information Carbon tracking information | .xlsx (compliant to BS 1192-4:2014) .xlsx (basic spreadsheet) .xlsx (basic spreadsheet) |
| Documentation in open format | .pdf |

Table 10: Information to be provided as each information milestone

Note: Native format refers to the file format which the Task Team’s BIM authoring software is designed to work with.

### Information Model review and authorisation

As per EIR section 7.2.3, the Design Lead (Appointed Party) during the design stages, and the Construction Lead (Appointed Party) upon appointment, will undertake a review of the information models shared by each Task Team in accordance with the project’s information production methods and procedures.

In doing this, the Design/Construction Lead (Appointed Party) will consider:

The Information Identification integrity – Information Container is checked for correct identification (ID) as defined in section 6.2 of the EIR

Compliance with the project Federation Strategy - Information Container is analysed to check that it has been appropriately divided into sets of information containers keeping the individual Information Container size under the 300mb limit specified in section 7.3 of the EIR

Compliance with BIM object authoring requirements – Information Container is analysed to check that BIM objects have been authored in accordance with section 6.2.2 of the EIR

The progression of the design and design information - Information Container is analysed to check for compliance with the level of information need required at a particular project stage. The development, completeness and validity of geometrical and non-geometrical information is reviewed against the Detailed BIM Responsibility Matrix

The development, completeness and validity of asset information which will be reviewed against the Asset Information Requirements

The development, completeness, and validity of the classification information within the Information Container, which will be reviewed against the requirements defined in section 6.3 of the EIR

The development, completeness and validity of and additional information requirements (e.g. performance attributes, carbon tracking information, etc.)

The level of quality and spatial coordination in the Federated Information Model.

- Figure 04. Information Review and Approval process

If the review is successful, the Design/Construction Lead (Appointed Party) will authorize each Information Model and instruct each Task Team to submit their information for SP Energy Networks acceptance within the project’s CDE.

If the review is unsuccessful, the Design/Construction Lead (Appointed Party) will reject the Information Model and instruct the Task Team to amend the information and re-submit for Design/Construction Lead (Appointed Party) authorization. Partial acceptance of the information to be exchanged can lead to coordination issues, therefore the Design/Construction Lead (Appointed Party) shall either authorise or reject the entire Information Model.

Errors or non-compliance must be corrected as a priority with agreed timescales. Actions must be distributed to all Project Stakeholders, via the CDE, by the Design Lead (Appointed Party) during the design stages, and by the Construction Lead (Appointed Party) following this.

### Information Model review and acceptance

As per EIR section 7.2.4, The Design Lead (Appointed Party) during the design stages, and the Construction Lead (Appointed Party) upon appointment, will undertake a review of the information models shared by each Task Team in accordance with the project’s information production methods and procedures.

In doing this, the Design/Construction Lead (Appointed Party) will consider:

The Information Identification integrity – Information Container is checked for correct identification (ID) as defined in section 6.2 of the EIR

Compliance with the project Federation Strategy - Information Container is analysed to check that it has been appropriately divided into sets of information containers keeping the individual Information Container size under the 300MB limit specified in section 7.3 of the EIR

Compliance with BIM object authoring requirements – Information Container is analysed to check that BIM objects have been authored in accordance with section 6.2.2 of the EIR

The progression of the design and design information - Information Container is analysed to check for compliance with the level of information need required at a particular project stage. The development, completeness and validity of geometrical and non-geometrical information is reviewed against the Detailed BIM Responsibility Matrix

The development, completeness and validity of asset information which will be reviewed against the Asset Information Requirements

The development, completeness, and validity of the classification information within the Information Container, which will be reviewed against the requirements defined in section 6.3 of the EIR

The development, completeness and validity of and additional information requirements (e.g. performance attributes, carbon tracking information, etc.)

The level of quality and spatial coordination in the Federated Information Model.

- Figure 03. Information review and acceptance process

- If the review is successful, the Design/Construction Lead (Appointed Party) will authorise each Information Model and instruct each Task Team to submit their information for SP Energy Networks acceptance within the project’s CDE.

- If the review is unsuccessful, the Design/Construction Lead (Appointed Party) will reject the Information Model and instruct the Task Team to amend the information and re-submit for Design/Construction Lead (Appointed Party) authorisation.

- Partial acceptance of the information to be exchanged can lead to coordination issues, therefore it is recommended that the Design/Construction Lead (Appointed Party) either authorises or rejects the entire Information Model.

Errors or non-compliance must be corrected as a priority with agreed timescales. Actions must be distributed to all Project Stakeholders, via the CDE, by the Design Lead (Appointed Party) during the design stages, and by the Construction Lead (Appointed Party) following this.

### Project Key Performance Indicators (KPIs)

SP Energy Networks have established the following Key Performance Indicators (KPIs). Full details of KPI including KPI measurement and KPI recommendations are included within the EIR. Task Team Managers are responsible for ensure the compliance of their 3D information models with these KPIs.

|  | Required compliance |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Key Performance Indicator | Concept & Initiation | Tender Design | VPC Design | Construction | Handover & Closeout |
| Level of geometric detail | 80% | 80% | 80% | 80% | 80% |
| Non-geometric information | n/a | 95% complete 80% accurate structure | 95% complete 90% accurate structure | 95% complete 95% accurate structure | 95% complete 95% accurate structure |
| Individual model quality | 70% | 80% | 85% | 95% | 95% |
| Federated model coordination | 70% | 80% | 85% | 95% | 95% |
| Programme delivery | n/a | 100% | 100% | 100% | 100% |

Table 11: Required compliance of Key Performance Indicators (KPI) by project stage

## Identification (ID) conventions

### Information container identification (ID)

Each information container shall have a unique ID, based upon SP Energy Networks’ Naming Convention Guidance document (doc reference), and make us of agreed ‘revision’ and ‘status codes’. For convenience this is replicated in part below, however the original reference retains preference.

| Project | Originator | Volume/ System | Level/ Location | Type | Role | Number |
| --- | --- | --- | --- | --- | --- | --- |
| TBC | TTD | XX | ZZ | SP | Z | 0000001 |
| Example: TBC-TTD-XX-ZZ-SP-Z-000001 |  |  |  |  |  |  |

Table 12: Information container identification (ID)

Note: If an information container is removed or exported from the common data environment, then the additional fields 'suitability' and 'revision', separated by a delimiter, should be added to its ID as a suffix.

#### Agreed originator codes

The following codes shall be used within information container IDs to identify the originator. A list of agreed codes for all other fields may be found within the Naming Convention Guidance of the Project Information Protocol.

| Originator (task team) | Code |
| --- | --- |
| TBC | TBC |

Table 13: Agreed originator codes

### Object identification (ID)

All objects within information models shall be identified in accordance with the BS 8541 series. For convenience the project’s interpretation of object identification (ID) is replicated in part below, however the original reference retains preference and is applicable in full.

| Object identification for classified objects |  |  |
| --- | --- | --- |
| Source | Type | Subtype/Product |
| Originator | First level of specialisation Use defined IFC types | Additional specialisation Use CamelCase to combine multiple subtype specialisms |
| Example: CIV_PipeSegment_110mmDrainageSewer |  |  |

Table 14. Object identification convention

### Location identification

All levels, zones and spaces shall be uniquely identified.

#### Levels

Codes for levels IDs shall match the project’s Information Management protocol (doc reference)

#### Spaces

Codes for space IDs shall follow the convention in Table 15 below. For consistency task teams shall adopt space IDs matching those generated by Responsible party.

| Space identification |  |
| --- | --- |
| Level | Integer |
| Code of the associated level on which a space is located | 3-digit integer unique to that level |
| Example: 01-001 |  |

Table 15: Space identification convention

#### Zones

Codes for zone IDs shall follow the convention in Table 16 below.

| Zone identification |  |  |
| --- | --- | --- |
| Originator | Type | Description |
| Originator (task team) code | One of: Circulation FireAlarm HistoricalPreservation Lighting Occupancy Ventilation | Plain language description of the zone. Use CamelCase to combine multiple subtype specialisms |
| Example: TTD-FireAlarm-WestContainmentArea |  |  |

Table 16: Zone identification convention

### Systems

Codes for system IDs shall follow the convention in Table 17 below.

| System identification |  |  |
| --- | --- | --- |
| Originator | Description | Integer |
| Originator (task team) code | Plain language description of the zone. - Use CamelCase to combine multiple subtype specialisms | 3-digit integer unique to that originator and description |
| Example: TTD-HighVoltagePower-001 |  |  |

Table 17: System identification convention

## Classification of information

Classification is required for all information within information models and shall be in accordance with Uniclass 2015.

Classification tables shall use as a minimum the version appearing in the table below. Tables may be accessed at this location: https://www.thenbs.com/our-tools/uniclass-2015

| Uniclass 2015 Table | Version | Publish Date |
| --- | --- | --- |
| Ro - Roles | V1.4 | January 2021 |
| En - Entities | V1.19 | January 2021 |
| SL – Space/locations | V1.19 | January 2021 |
| Ss – Systems | V1.21 | January 2021 |
| Pr - Products | V1.21 | January 2021 |

Table 18: Applicable Uniclass 2015 classifications

Where Task Teams are authoring 3D information models using Revit, the population of Uniclass2015 within the Information Models shall be achieved using the Autodesk Classification Manager for Revit add-in: http://www.biminteroperabilitytools.com/classificationmanager.php.

The Autodesk Classification Manager for Revit add-in will populate Uniclass2015 information within attributes named as per the list below. Where a Task Team is not using Revit, or information exists outside the Revit file, Information Originators shall replicate the naming of these attributes within their chosen software. This shall ensure consistent information delivery.

Classification.Roles.Number

Classification.Roles.Description

Classification.Facility.Number

Classification.Facility.Description

Classification.Space.Number

Classification.Space.Description

Classification.Uniclass.Ss.Number

Classification.Uniclass.Ss.Description

Classification.Uniclass.Pr.Number

Classification.Uniclass.Pr.Description.

## Risk Register

The Delivery Team’s risk register for risks associated to the timely delivery of information in accordance with the Appointing Party’s EIR has been incorporated within the project’s main risk register, <CDE Reference>.

Review of the risk register shall form an agenda item at each BIM coordination meeting. To facilitate this, all risks related to fulfilling the EIR shall be classified as ‘BIM’, so that they may be easily filtered.

## Task information Delivery Plan

As per EIR section 6.8 each Task Team shall establish a TIDP and maintain this throughout their appointment. Each Task Team Manager is responsible for ensuring their Task Team’s TIDP remains current.

On each occasion the TIDP undergoes revision, the Task Team Manager shall upload the new version to the following CDE location and notify the Lead Appointing Party.

Location of TIDPs: <CDE Reference>

# INFORMATION PRODUCTION METHODS AND PROCEDURES

## Common Data Environment

| CDE | BIM 360 |
| --- | --- |
| CDE location | <CDE Reference> |

Table 19: Details of the Common Data Environment

The purpose of the CDE is to provide SP Energy Networks and Project Stakeholders with a secure, central, accessible resource for collecting, managing and disseminating each information container through a managed process. A CDE ‘workflow’ describes the processes to be used and a CDE “solution” can provide the technology to support those processes.

On this project, the current CDE is owned by SP Energy Networks. The CDE will be the repository for the PIM for the project lifecycle. The Lead Appointing Party CDE Information Manager has established the project’s common data environment (CDE) to serve the overall requirements of the project and to support the collaborative production of information. All stakeholder shall comply with the information management processes outlined in the Information Management Protocol.

Project Stakeholders are responsible for storing and maintaining a copy of all their project information in a secure, stable location within their own organisation and must make information available if requested, over the CDE. SP Energy Networks must have access to both native and exchange information models stored in the CDE at any point during the project.

As per the requirement contained within the EIR models must be issued within three working days of an RFI requesting this information.

### Trialling of the CDE

To trial the exchange of information models, the Lead Appointed Party, with support from the Lead Appointed Party CDE Information Manager will facilitate the initial sharing and linking of project models issued via the CDE. The process for this will be as follows.

The Lead Appointed Party will generate a ‘Shared levels and grids’ model as per section 8.3.1

The Lead Appointed Party will issue their model in both native format and exchange information format using the shared CDE workflow

All task teams will access the ‘Shared levels and grid’ model and use this as a reference to create their own information model

All task teams will issue their information models to the CDE using the shared workflow.

Upon completion of the trial(s) the Lead Appointed Party and Appointing Party Information Manager will report outcomes to the Appointing Party CDE Information Manager.

## Collaboration process

### BIM Meetings

To establish and maintain a collaborative environment the following meetings have been identified. The Lead Appointed Party will notify via email invite any individuals required to attend.

|  |  | Project Stage |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Meeting | Description | Concept & Initiation | Tender Design | VPC Design | Construction |
| BIM kick-off workshop | Delivery team playback of the BIM Execution Plan (BEP) Freq: Start of stage and upon award of new appointments | ✓ |  |  | ✓ |
| BIM coordination meeting | Spatial and data coordination of federated BIM. Resolution responsibilities agreed Freq: Fortnightly | ✓ | ✓ | ✓ | ✓ |
| BIM lessons learnt meeting | Review of project issues and solutions Freq: Quarterly |  | ✓ | ✓ | ✓ |
| Supply chain meetings | Workshop meetings are held to discuss competence, implementation and best practice. Freq: As required |  | ✓ | ✓ | ✓ |

Table 20: BIM meetings

### Coordination and clash detection process

All task teams are to issue models a minimum of five working days prior to Design Team/BIM Coordination meetings.

#### Coordination and clash detection

All task teams are responsible for clash detecting their own models prior to issue in accordance with the requirements of the Information Delivery Strategy. The table below details individual’s responsibility to input to the delivery of a ‘clash free’ Project Information Model.

|  |  |  | Activity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Individual Responsible | Role | Model Federation | Individual model checks | Federated model checks | Clash management | Clash resolution |
| TBC | Lead Appointed Party | ✓ |  | ✓ | ✓ | ✓ |
| TBC | Task Information Manager |  | ✓ |  |  | ✓ |
| TBC | Task Information Manager |  | ✓ |  |  | ✓ |
| TBC | Task Information Manager |  | ✓ |  |  | ✓ |
| TBC | Appointing Party Information Manager | ✓ |  |  | ✓ |  |

Table 21: Clash detection and resolution responsibilities

#### Coordination and clash detection rulesets

To ensure compliance with the project’s Key Performance Indicators the following rulesets have been agreed by the Appointing Party and Delivery Team

| Category A | Category B |
| --- | --- |
| TBC | TBC |

Table 22: Clash detection rulesets

#### Clash Detection Programme/Frequency

The following table details the cyclical coordination process that must be followed by all stakeholders.

| Role Abbreviations for use in Table 23 below |  |  |
| --- | --- | --- |
| LAP Lead Appointed Party | APIM Appointing Party Information Manager | TIM Task Team Interface Manager |
| TTM Task Team Manager | IO Information Originator |  |

|  |  | Week 1 |  |  |  |  | Week 2 |  |  |  |  | Week 3… |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Action | Resp. | M | T | W | T | F | M | T | W | T | F | M | T | W | T | F |
| Development of 3D information models | IO | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ | ✓ |
| Internal QA/QC check | TIM |  |  |  | ✓ |  |  |  |  |  |  |  |  |  | ✓ |  |
| Issue information exchange to CDE | TTM |  |  |  |  | ✓ |  |  |  |  |  |  |  |  |  | ✓ |
| Clash detection and report | LAP |  |  |  |  |  | ✓ | ✓ |  |  |  |  |  |  |  |  |
| Review report & update models | TTM |  |  |  |  |  |  |  | ✓ | ✓ | ✓ |  |  |  |  |  |
| BIM coordination meeting | All |  |  |  |  |  |  |  |  |  |  | ✓ |  |  |  |  |
| Assign unresolved actions | LAP |  |  |  |  |  |  |  |  |  |  |  | ✓ | ✓ |  |  |
| Update models | IO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 23: Programme of clash detection process

Each Task Team Interface Manager (TIM) must download and review the clash detection report produced by the Lead Designer (stored on the CDE) and resolve all errors or noncompliance (where possible) prior to attending the subsequent Design Team/BIM Coordination meeting.

## Generation, review, approval and authorisation of information

### Coordinate System

To eliminate compatibility issues arising from discrepancies between coordinate systems the Design Lead Appointed Party will issue a ‘Shared levels and grids’ model to the CDE. This information model may be found at the location below;

| Container ID | <Enter information container identity> |
| --- | --- |
| CDE location | <Enter CDE location> |

All task teams shall reference the ‘Shared levels and grid’ model when establishing their own information model’. Values for project base point, survey point, internal origin and rotation from true north shall not be altered at any time. For record, the common coordinate system defined by the ‘Shared levels and grids’ model is as below.

| Coordinate | Northing (m) | Easting (m) | Elevation (m) | Rotation from True North (°) |
| --- | --- | --- | --- | --- |
| Project base point | ?.?? | ?.?? | ?.?? | ?.?? |
| Survey point | ?.?? | ?.?? | ?.?? | ?.?? |

Table 24: Coordinate system

### Model units of measurement

All information models and associated design information shall be produced and communicated using;

The metric system

A scale of 1:1, giving distance in units of millimetres

### Agreed modelling tolerances

Acceptable project modelling tolerances are recorded below and are to be used during spatial coordination. These tolerances do not represent building tolerances for construction or manufacture.

| DD = Design Development | DA = Dimensionally Accurate | AP = Approximate Position | G = Geometry Tolerance |
| --- | --- | --- | --- |

| Modelling Tolerances (Not Construction Tolerances) | Concept & Initiation | Tender Design | VPC Design | Construction | Handover |
| --- | --- | --- | --- | --- | --- |
| Category A |  |  |  |  |  |
| Classification 1 |  |  |  |  |  |
| Classification 2 |  |  |  |  |  |

Table 25: Agreed modelling tolerances

### Drawing sheet

All drawing sheets shall be derived from within the 3D information model to ensure compliance with the EIR. No modifications to drawings shall be performed in 2D modelling software to ensure accuracy and coordination are maintained.

In all cases, information held in published 3D models takes preference over 2D models.

#### Drawing sheet template

To aid consistency of information production the Design Lead Appointed Party has issued a project specific ‘Drawing sheet template’ to the CDE. This template may be found at the location below;

| Container ID | <Enter information container identity> |
| --- | --- |
| CDE location | <Enter CDE location> |

The drawing sheet template has been structured to include;

Information container title

Information container ID

Revision

Project title

Project code

Appointed party’s logo

Lead Appointed Party’s logo

## Security and distribution of information

### Security of information

All project stakeholders are required to adhere to the processes outlined in BS ISO 19650-5:2020.

This requires all project deliverables shall be shared through the CDE only. Project stakeholders must independently take appropriate measures to ensure the secure use of information within their organisation and supply chain.

In the event of a breach / incident compromising the security of project information, the discovering stakeholder is required to immediately notify the Build Asset Security Manager.

### Health and Safety

BIM information will be made available to the Principal Designer/Construction by all Task Teams and Stakeholders for identifying and managing risk. The Principal Designer/Contractor and Delivery Team are obliged to inform of potential hazards and risks to the Appointing Party and Appointing Party Information Manager.

The below process dictates the project strategy for the how the models will be utilised to capture the risks in line with the requirements of CDM (Health and Safety). The procedures detailed below assure that information and geometry related to H&S issues are generated within the BIM Models, improving the way this information is managed throughout the whole life cycle of the asset.

#### CDM

The Construction Design and Management Regulations 2015 place statutory requirements on all designers in the construction process.

CDM aims to improve health and safety in the industry by helping you to:

- Sensibly plan the work so the risks involved are managed from start to finish.

- Have the right people for the right job at the right time.

- Cooperate and coordinate your work with others.

- Have the right information about the risks and how they are being managed.

- Communicate this information effectively to those who need to know.

- Consult and engage with workers about the risks and how they are being managed.

Refer to the Health and Safety Executive (http://www.hse.gov.uk) for more detail.

Project Stakeholders have a responsibility under the legislation to identify risks or hazards associated with the project and these are combined in a risk register with those generated by the other team members.

#### Risk Cubes

Each Task Team must position a ‘risk cube’ wherever a residual risk is present within their scope of design/construction. This should be linked via use of the Risk ID parameter to the project’s health and safety register.

A template risk cube component is located <CDE Reference>.

The following parameters are applied to the risk cube:

Risk Number

Risk Date

Risk Generated By

Risk Period

Person at Risk

Hazard Description

Hazard Mitigation

Residual Risk

Responsible Party

Risk Status

Risk Active

Closed Risk

Residual Risk Passed On

Risk Group

#### Risk Register and Schedules

All Project Stakeholders will produce a Discipline Master Risk Schedule that is created in the model to manage the risks identified in the project. It is the responsibility of the Task Team Manager to edit and verify the schedule so that it meets the project requirements and those of the CDM Regulations.

As well as a Discipline Master Risk Schedule, other sub schedules that are filtered by the parameter Risk Group will be required. The idea is to group risks associated with a set of Drawings and include this limited schedule on the sheets.

For example if a series of risks are associated with windows and Falling from a height. All those risks can be identified with a common code and the schedule that just includes those risks may be included on all the elevation sheets.

#### Tagging

The project will utilise the project “risk tag” to tag risks in a view. The tag ‘ORIG_RiskTag_RiskNumber-RiskStatus’ displays the Risk Number and the Risk Status. The Information Originator placing the Object will have to change the tag name and replace ORIG with their organisation reference.

#### Visibility

It is important to have the ability to conceal the risk markers easily in all general views. The 3D Element is a subcategory of the Generic Models category. Task Teams may wish to create CDM specific plans and elevations etc which suppress building objects and bring risk identification to the forefront.

#### Non Location Specific Risks

It may become necessary to identify and track a risk or hazard that has no obvious specific location. For these items the risk markers should be placed in a location to be agreed by Project Stakeholders. The Master Schedule should then be set to show the Object in all phases.

## Legacy information

### Reference information, shared resources and object library

Any information, shared resources and object library received from the Appointing Party has been made available at the CDE location below. Task teams should endeavour to supplement this

| Container ID | <Enter information container identity> |
| --- | --- |
| CDE location | <Enter CDE location> |

The following information is only available in hard copy. Access to hard copy information may be requested by contacting the Appointing Party.

### Survey

Where task teams are required to, or otherwise capture existing condition information, either via traditional measurement, laser capture or otherwise, task teams shall follow the guidance set out in BS 5964-3:1996.

To ensure captured survey data is coordinated and useable, task teams shall comply with the format below;

| Survey Method | Delivery Format | Survey Origin | Details / Notes |
| --- | --- | --- | --- |
| Point Cloud | TBC | TBC | TBC |
| Other | TBC | TBC | TBC |

Table 26: Defined survey formats

# FEDERATION STRATEGY

## Information breakdown strategy

The table below defines the strategy for how the PIM shall be divided into sets of information containers. This shall be updated as new task teams are appointed.

| Discipline | Volume/System description | Information model |
| --- | --- | --- |
| TBC | <Enter 2 digit code> | <Enter model identifier> |
| TBC |  |  |
| TBC |  |  |

Table 27: Information breakdown strategy

Note: Individual information models must not exceed 250mb to ensure that file sizes are practical for upload and download with the specified IT infrastructure. Where a task team’s information model is at risk of exceeding this limit they shall notify the Appointed Party CDE Information Manager, Appointing Party Information Manager and Appointing Party and respond with any mitigating actions deemed necessary.

## Federation process

The Design Lead Appointed Party is responsible for accessing information provided by all task teams and federating their 3D information models into a single multidisciplinary information model. Upon their appointment the Construction Lead will acquire this responsibility. As a minimum requirement, the federated model will be updated monthly and issued onto the CDE throughout the project duration unless explicitly agreed with the Lead Appointed Party.

The federation process will be as follows;

The responsible party will create a federate model using the exchange format information models uploaded to the CDE

The federated model will be created using the software outlined in section 10.1 with the resultant file published to the CDE.

# SOFTWARE, HARDWARE AND IT INFRASTRUCTURE

## Software

The agreed software, version and service pack for each discipline are as defined below. Only software identified below shall be used to author, coordinate and validate information models. Any update is to be explicitly agreed by the Appointing Party and Appointing Party Information Manager.

| Discipline / Organisation | Software & Version | Service Pack |
| --- | --- | --- |
| Information Container Management |  |  |
| All | BIM 360 | - |
| Design Authoring |  |  |
| Design Delivery Team | Autodesk Revit YYYY | TBC |
| TBC | TBC | TBC |
| TBC | … | … |
| Coordination & clash detection |  |  |
| All (internal checks) | Autodesk Navisworks Manage 2021 | - |
| Lead Appointed Party (federated checks) | Autodesk Navisworks Manage 2021 | - |
| Data verification |  |  |
| TBC | TBC | TBC |
| Construction Programming |  |  |
| TBC | TBC | TBC |

Table 28: Agreed software

## Hardware and IT Infrastructure

All Project Stakeholders shall deploy hardware and IT infrastructure appropriate to the fulfilment of the Exchange Information Requirements and this BIM Execution Plan.

This shall be achieved as each Project Stakeholder sees fit, with consideration to the project’s methods and procedures, technical requirements, and security requirements.

Appendix A – Abbreviations and glossary of terms

Abbreviations

| Term | Expansion |
| --- | --- |
| AIR | Asset Information Requirements |
| BEP | BIM Execution Plan |
| BIM | Building Information Modelling |
| CDE | Common Data Environment |
| CDM | Construction (Design and Management) |
| EIR | Exchange Information Requirements |
| FM | Facilities Management |
| IFC | Industry Foundation Classes |
| KPI | Key Performance Indicator |
| MIDP | Master Information Delivery Plan |
| PIM | Project Information Model |
| PIR | Project Information Requirements |
| TBC | To be confirmed |
| TIDP | Task Information Delivery Plan |

Glossary of terms

| Term | Definition |
| --- | --- |
| Terms relating to assets and projects |  |
| Appointed Party | Provider of information concerning works, goods or services |
| Lead Appointed Party | Appointed party who has foremost responsibility amongst the delivery team for the management of the BIM process during the applicable stage. Please note that our understanding of ‘Lead’ in this manner is not to denote any contractual relationship hierarchy between Appointed Parties. |
| Appointing party | Receiver of information concerning works, goods, or services from a lead appointed party |
| Delivery team | Lead Appointed Party and the Appointed Parties Where appropriate this term is prefixed with ‘Design’ or ‘Construction’ to indicate responsibility. |
| Task team | Individuals assembled to perform a specific task |
| Key decision point | Point in time during the life cycle when a decision crucial to the direction or viability of the asset is made |
| Terms relating to information management |  |
| Information | Reinterpretable representation of data in a formalised manner suitable for communication, interpretation, or processing |
| Information requirements | Specification for what, when, how and for whom information is to be produced |
| Exchange Information Requirements (EIR) | Information requirements in relation to an appointment |
| Asset Information Requirements (AIR) | Information requirements in relation to the operation of an asset |
| Information exchange | Act of satisfying an information requirement or part thereof |
| Information model | Set of structured and unstructured information containers |
| Asset information model | Information model relating to the operational phase |
| Project information model | Information model relating to the delivery phase |
| Federation | Creation of a composite information model from separate information containers |
| Federated Information Model | A set of information containers combined into a single information model |
| Information container | Named, persistent set of information retrievable from within a file, system, or application storage hierarchy |
| Common data environment (CDE) | Agreed source of information for any given project or asset, for collecting, managing, and disseminating each information container through a managed process. A CDE ‘workflow’ describes the processes to be used and a CDE ‘solution’ can provide the technology to support those processes |
| Level of Information Need | Framework which defines the extent and granularity of information |
| General terms |  |
| Attribute | A specification that defines a property of an object |
| COBie | A specialised and structured information exchange used to transfer asset information to be used during the operation phase. |
| Drawing | An information model presenting a static, graphical representation of part or all of a project or asset |
| Native information model | The primary data format of a set of structured and unstructured information containers |
| Object or Component | Detailed information that defines a product and geometry that represents that product’s physical characteristics. |
| Shared levels and grids container | Information container defining the common coordinate system to be used across the project. This defines, but is not limited to, the Project Base Point, Survey Point and project common levels and grids. |
| Space | Limited three-dimensional extent defined physically or notionally |
| Zone | A group of spaces, partial spaces, or other zones |
| 4D | A 3D representation of an asset with time-related information. This information can be used to obtain accurate programme information and visualisations showing how the project develops sequentially |

Appendix B - Reference summary

| Reference | Title | Abstract |
| --- | --- | --- |
| BIM according to ISO 19650 |  |  |
| BS EN ISO 19650-1:2018 (Part 1 – Concepts and principles) | Organization and digitization of information about buildings and civil engineering works, including building information modelling (BIM) – Information management using building information modelling. | Sets out the recommended concepts and principles for business processes across the built environment sector in support of the management and production of information during the life cycle of built assets (referred to as “information management”) when using building information modelling (BIM). |
| BS EN ISO 19650-2:2018 (Part 2 – Delivery phase of the assets) | Organization and digitization of information about buildings and civil engineering works, including building information modelling (BIM) – Information management using building information modelling. | Enables an Appointing Party to establish their requirements for information during the delivery phase of assets and to provide the right commercial and collaborative environment within which (multiple) Appointed Parties can produce information in an effective and efficient manner. |
| PAS 1192-3:2013 | Specification for information management for the operational phase of assets using Building Information Modelling | Provides guidance to Asset Managers on how to integrate the management of information across the longer term activity of asset management with the shorter term activity of asset construction for a portfolio of assets. |
| BS 1192-4:2014 | Collaborative production of information Part 4: fulfilling exchange information requirements using COBie | Outlines the UK usage of COBie, an internationally agreed information exchange schema for exchanging facility information between the Appointing Party and the supply chain. |
| PAS 1192-5:2015 | Specification for security-minded building information modelling, digital built environments and smart asset management | Specifies requirements for security-minded management of BIM and digital built environments. It outlines the cyber-security vulnerabilities to hostile attack when using BIM and provides an assessment process to determine the levels of cyber-security for BIM collaboration which should be applied during all phases of the site and building lifecycle. |
| PAS 1192-6:2018 | Specification for collaborative sharing and use of structured Health and safety information using BIM | Specifies requirements for the collaborative sharing of structured H&S information throughout the project and asset life-cycles. This supports the development of structured H&S information for all construction projects progressively from the outset. |
| BIM according to ISO 19650 enablers |  |  |
| BS 8541-1:2012 | Library objects for architecture, engineering and construction Part 1: Identification and classification – Code of practice | Provides recommendations for defining format and content for library objects to support project briefing, design, tendering, construction and management of built assets. |
| BS 8541-2:2011 | Library objects for architecture, engineering and construction Part 2: Recommended 2d symbols of building elements for use in building information modelling | Provides guidance and recommendations and establishes rules for the design of symbols and other graphic conventions used in drawings for the construction industry. |
| BS 8541-3:2012 | Library objects for architecture, engineering and construction Part 3: Shape and measurement | Provides guidance and recommendations for shape and measurement of construction library objects used in building construction and facility domains, setting levels of detail and representations of measurement. |
| BS 8541-4:2012 | Library objects for architecture, engineering and construction Part 4: Attributes for specification and assessment – Code of practice | Provides guidance and recommendations for the application of construction objects integrated into BIM working, defining the level of information for specific uses including specifying desired outcomes and selection of products. Covers common and specific attributes and can be used for the assessment of expected impacts. |
| BS 8541-5:2015 | Library objects for architecture, engineering and construction Part 5: Assemblies – Code of practice | Provides guidance and recommendations for sharing sub-models of combinations of components and spaces, covering naming, classification and nesting. |
| BS 8541-6:2015 | Library objects for architecture, engineering and construction Part 6: Product and facility declarations – Code of practice | Provides guidance and recommendations for sharing information from product declarations, labelling and environmental tables. |
| NBS BIM Toolkit | NBS BIM Toolkit | The free-to-use NBS BIM Toolkit will benefit both public and private sector construction projects. It provides step-by-step help to define, manage and verify responsibility for information development and delivery at each stage of the asset lifecycle. This toolkit is an indispensable way of delivering projects to meet the requirements of BIM according to the ISO 19650 series. |
| Uniclass2015 | Uniclass2015, the classification for the Construction Industry | Provides a structured approach to classifying the building information by organising information based upon common characteristics. |
| BS 5964-3:1996 | Building setting-out and measurement. Check-lists for the procurement of surveys and measurement services | Relates to checklists for surveying and setting-out services that may be required during the various stages of the development and construction of buildings and ancillary works |
| Project protocols |  |  |
| Information Management Protocol | Information Management Protocol | The purpose of this document is to have a controlled and standardised approach for collaborating all project information and documentation. This ensure that all project related information is shared and distributed in a manner that is accurate, comprehensive, reliable and systematic, resulting in access to the right information at the right time. |
| Exchange Information Requirements | Exchange Information Requirements | Document specifying the Employers requirements for exchange of information throughout all stages of the project. |

Appendix C - Detailed (BIM) Responsibility Matrix

For the Detailed (BIM) Responsibility Matrix Template refer to the following document:

XXX-TTD-XX-XX-SP-K-000004

Appendix D – Master Information Delivery Table

For the Master information Delivery Plan refer to the following document:

XXX-TTD-XX-XX-PL-K-000002

Appendix E – Asset Information Requirements

For the Asset Information Requirements (AIR) refer to the following document:

XXX-TTD-XX-XX-SP-K-000003

Appendix F – IFC Export Settings

For the Asset Information Requirements (AIR) refer to the following document:

XXX-TTD-XX-XX-PL-K-000003